## SOLUTION OF THIN RECTANGULAR PLATES

## WITH VARIOUS BOUNDARY CONDITIONS

## Mykhaylo Delyavskyy1, Krystian Rosiński2, Justyna Sobczak-Piąstka1,
Dariusz Buchaniec3, Yuriy Famulyak4

1Bydgoszcz University of Science and Technology, Faculty of Civil, Architecture and Environmental Engineering, Al. prof. S. Kaliskiego 7, 85-796 Bydgoszcz, Poland,
email: delyavmv@pbs.edu.pl, justynas@pbs.edu.pl

2PUH LEM-BUD Sp. z o.o., ul. Okólna 128, 87-100 Toruń, Poland,
email: krystian.rosinski@gmail.com

3University of Economy in Bydgoszcz, Institute of Building and Engineering Design,
ul. Garbary 2, 85-229 Bydgoszcz, email: dariusz.buchaniec@byd.pl

4Lviv National University of Nature Management, Building and Architecture Faculty, 80381, V. Velykoho Str., Dublyany, Lviv region, Ukraine, email: yufam@ukr.net

# Abstract

The paper presents an analytical-numerical method to solve thin rectangular plate constructions loaded by continuous load. The developed technique is based on the model of continuum material and allows to reduce the structure modeling to a solution that consists of kinematic and static boundary conditions written in the separate points at the plate contour. This has led to obtaining elegant and compact analytic solution capable of easy numerical evaluation. The analysed problem is an extension of the description presented in the paper [1].

In contrast to FEM, the suggested technique uses essentially fewer equations to solve the structure and provides better accuracy of solution. To solve a problem, the FEM subdivides structure into many smaller parts that are called finite elements, while suggested technique uses large separate parts called macroelements [2,3].

| TABLE 1. The variants of rectangular plates with various boundary conditions, where S – simply supported edge, C – clamped edge, F – free edge. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SS | SC | SF | CC | CF | FF |
| SS | SSSS | SSSC | SSSF | SSCC | SSCF | SSFF |
| SC |  | SCSC | SCSF | SCCC | SCCF | SCFF |
| SF |  |  | SFSF | SFCC | SFCF | SFFF |
| CC |  |  |  | CCCC | CCCF | CCFF |
| CF |  |  |  |  | CFCF | CFFF |
| FF |  |  |  |  |  | FFFF |

Computational model is implemented in author’s computer program which uses automatic differentiation (AD). The program allows to easily modeling configuration of the plate, to write the various types of boudary conditions and provide high accuracy of solution with efficiency of calculations compared to numerical methods. The reliability of the obtained results was confirmed by their consistency and comparison with the results obtained with other methods.

# References

- Delyavskyy M., Rosiński K.: The New Approach to Analysis of Thin Isotropic Symmetrical Plates, Applied Sciences. 2020; 10(17):5931. https://doi.org/10.3390/app10175931

- Delyavskyy M., Rosiński K.: Solution of non-rectangular plates with macroelement method, AIP Conference Proceedings 1822, 020005 (2017) https://doi.org/10.1063/1.4977679

- Delyavskyy M., Rosiński K., Zdolbicka N., Bilash O.: Macroelement analysis of thin orthotropic polygonal plate resting on the elastic Winkler’s foundation, AIP Conference Proceedings 2077, 020014 (2019) https://doi.org/10.1063/1.5091875

Keywords: thin plate, analytical-numerical method, macroelements